-------------------------------------------------------------------------------

--

--   Floating Point Adder/Subtractor Benchmark: Test Documentation

--

-- Source:  Patterson, David A., and Hennessy, John L.,

--               "Computer architecture: a quantitative approach".

--		  San Mateo, CA: Morgan Kaufman Publishers, 1990,

--                Appendix A, p. 12-20

--

-- Author: Bob McIlhenny

--         University of California, Irvine, CA 92717

--

--    based on an original design by:

--    Marc Rose, Intel Corporation, Floating Point Adder/Subtractor Version 3

--

-- Written on June 9, 1993

--

-- Modified by Jesse Pan on Dec 7, 1993

-------------------------------------------------------------------------------

TESTING THE Floating Point Adder/Subtractor PROCESS.

PORTS

clk		input

op1_sign	input

op1_exp		input

op1_mant	input

op2_sign	input

op2_exp		input

op2_mant	input

res_sign	output

res_exp		output

res_mant	output

operation	input

flags		output

Details about these ports are mentioned in the adder.doc file.

OVERALL TESTING STRATEGY

In testing the Adder/Subtractor, the overall strategy is to test for typical

behaviors and exhaustive testing of boundary conditions, performing operations

among the following significant operands.

In addition to nominal cases and modes of operations, extensive tests on

boundary cases (e.g. Not-a-Number, +/- Infinity) were done.

TESTING THE ADDER/SUBTRACTOR

Now, we will chart the testing of the Floating Point Adder/Subtractor.

*************************************************************************

*								        *

* TEST VECTORS FOR OUTPUT PRODUCED FROM OPERATION PERFORMED ON OPERANDS *

*								        *

*************************************************************************

IEEE format for Nominal numbers ->

sign = 0 or 1,  0 <= exponent <= 254 ,  mantissa = any 23-bit vector

Note: if exponent = 0 and mantissa is all zeros, this is not a nominal number,

it is the special representation of zero.

The follow table shows the typical test vectors and the result of operations.

The LSB in the MANT field are octave; the remaining five digit are hexadecimal.

****************************************************************************

*    |  OPERAND 1   | OPERAND 2    |  RESULT      |                        *

*TEST|   |   | Hex  |   |   | Hex  |   |   | Hex  |        OPERATION       *

*  # |SGN|EXP| MANT |SGN|EXP| MANT |SGN|EXP| MANT |                        *

*----|---|---|------|---|---|------|---|---|------|------------------------*

*  1 | 1 |125|000000| 1 |126|000000| 1 |126|400000|-0.25 + (-0.5) = -0.75  *

*----|---|---|------|---|---|------|---|---|------|------------------------*

*  2 | 1 |126|400000| 0 |128|000000| 1 |128|300000|-0.75 + (-2.0) = -2.75  *

*----|---|---|------|---|---|------|---------------------------------------*

*  3 | 1 |126|000000| 0 |124|000000| 1 |125|400000|-0.5 + 0.125  = -0.375  *

****************************************************************************

* The following table contents show the sign of the numbers and the sign of the

of the exponents after the bias are subtracted.

* For nominal add and subtract operations, the TEST # column every sign

combination of test vector, (ie positive/negative of sign/exponent),

are tested for four rounding situation. The rounding situation breakdown as

following:

* (r: round bit	   s: sticky bit   L: Lsb of preliminary result mantissa

X: don't care)

(TEST) #.A  - OPERATION(ADD/SUBTRACT) without rounding(r=0, s=X, L=X)

#.B  - OPERATION with rounding(r=1, s=1, L=X)

#.C  - OPERATION with round/even/truncate(r=1, s=0, L=0)

#.D  - OPERATION with round/even/plus 1  (r=1, s=0, L=1)

* For the boundry conditions test the rounding will not be tested.

NOMINAL ADD OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |  OPERATION RESULT

#  | SIGN/MANT| EXPONENT | SIGN/MANT| EXPONENT |

-----|----------|----------|----------|----------|------------------------

| negative | negative | negative | negative |

-----|----------|----------|----------|----------|------------------------

1.A |  100000  |   125    |  000000  |   126    | - [126 	400000]

-----|----------|----------|----------|----------|------------------------

2.B |  000005  |   123    |  000000  |   126    | - [126       100001]

-----|----------|----------|----------|----------|------------------------

3.C |  000004  |   123    |  000000  |   126    | - [126       100000]

-----|----------|----------|----------|----------|------------------------

4.D |  000004  |   123    |  000001  |   126    | - [126       100002]

-----|----------|----------|----------|----------|------------------------

| negative | negative | negative | positive |

-----|----------|----------|----------|----------|------------------------

5.A |  400000  |   126    |  000000  |   128    | - [128	300000]

-----|----------|----------|----------|----------|------------------------

6.B |  000009  |   126    |  000001  |   130    | - [130       080002]

-----|----------|----------|----------|----------|------------------------

7.C |  000008  |   126    |  000004  |   130    | - [130       080004]

-----|----------|----------|----------|----------|------------------------

8.D |  000008  |   126    |  000005  |   130    | - [130       080006]

-----|----------|----------|----------|----------|------------------------

| negative | negative | positive | negative |

-----|----------|----------|----------|----------|------------------------

9.A |  000000  |   126    |  000000  |   124    | - [125 	400000]

-----|----------|----------|----------|----------|------------------------

10.B |  7ffff3  |   122    |  000001  |   126    | + [125       600004]

-----|----------|----------|----------|----------|------------------------

11.C |  7ffffc  |   122    |  000002  |   126    | + [125       600004]

-----|----------|----------|----------|----------|------------------------

12.D |  7ffff4  |   122    |  000003  |   126    | + [125       600008]

-----|----------|----------|----------|----------|------------------------

| negative | negative | positive | positive |

-----|----------|----------|----------|----------|------------------------

13.A |  200000  |   126    |  000000  |   127    | + [125	400000]

-----|----------|----------|----------|----------|------------------------

14.B |  7fffe2  |   126    |  100002  |   129    | + [128       60000c]

-----|----------|----------|----------|----------|------------------------

15.C |  7fffe4  |   126    |  200003  |   129    | + [129       000006]

-----|----------|----------|----------|----------|------------------------

16.D |  7fffe4  |   126    |  200004  |   129    | + [129       000008]

-----|----------|----------|----------|----------|------------------------

| negative | positive | negative | negative |

-----|----------|----------|----------|----------|------------------------

17.A |  400000  |   127    |  000000  |   126    | - [128       000000]

-----|----------|----------|----------|----------|------------------------

18.B |  00000b  |   128    |  000034  |	  123    | - [128       04000d]

-----|----------|----------|----------|----------|------------------------

19.C |  00000b  |   128    |  000030  |   123    | - [128       04000c]

-----|----------|----------|----------|----------|------------------------

20.D |  00000c  |   128    |  000030  |   123    | - [128       04000e]

-----|----------|----------|----------|----------|------------------------

| negative | positive | negative | positive |

-----|----------|----------|----------|----------|------------------------

21.A |  300000  |   128    |  300000  |   128    | - [129   	300000]

-----|----------|----------|----------|----------|------------------------

22.B |  00000b  |   128    |  000001  |   132    | - [132       080002]

-----|----------|----------|----------|----------|------------------------

23.C |  000008  |   128    |  000006  |   132    | - [132       080006]

-----|----------|----------|----------|----------|------------------------

24.D |  000008  |   128    |  000007  |   132    | - [132       080008]

-----|----------|----------|----------|----------|------------------------

| negative | positive | positive | negative |

-----|----------|----------|----------|----------|------------------------

25.A |  400000  |   128    |  000000  |   126    | - [128 	200000]

-----|----------|----------|----------|----------|------------------------

26.B |  100001  |   129    |  7fff70  |   121    | - [129       0f0002]

-----|----------|----------|----------|----------|------------------------

27.C |  100002  |   129    |  7fff80  |   121    | - [129       0f0002]

-----|----------|----------|----------|----------|------------------------

28.D |  100003  |   129    |  7fff80  |   121    | - [129       0f0004]

-----|----------|----------|----------|----------|------------------------

| negative | positive | positive | positive |

-----|----------|----------|----------|----------|------------------------

29.A |  000000  |   129    |  000000  |   129    | + [0 	000000]

-----|----------|----------|----------|----------|------------------------

30.B |  020003  |   138    |  7fff21  |   130    | - [138       010004]

-----|----------|----------|----------|----------|------------------------

31.C |  020006  |   138    |  7fff80  |   130    | - [138       010006]

-----|----------|----------|----------|----------|------------------------

32.D |  020007  |   138    |  7fff80  |   130    | - [138       010008]

-----|----------|----------|----------|----------|------------------------

| positive | negative | negative | negative |

-----|----------|----------|----------|----------|------------------------

33.A |  000000  |   129    |  000000  |   125    | + [125 	000000]

-----|----------|----------|----------|----------|------------------------

34.B |  100020  |   110    |  7fffa6  |   106    | + [110       000026]

-----|----------|----------|----------|----------|------------------------

35.C |  100021  |   110    |  7fffa8  |   106    | + [110       000026]

-----|----------|----------|----------|----------|------------------------

36.D |  100022  |   110    |  7fffa8  |   106    | + [110       000028]

-----|----------|----------|----------|----------|------------------------

| positive | negative | negative | positive |

-----|----------|----------|----------|----------|------------------------

37.A |  400000  |   126	   |  000000  |   127    | - [125 	000000]

-----|----------|----------|----------|----------|------------------------

38.B |  7fff71  |   120    |  010005  |   128    | - [128       000006]

-----|----------|----------|----------|----------|------------------------

39.C |  0ffe80  |   120    |  100005  |   128    | - [128       0f7006]

-----|----------|----------|----------|----------|------------------------

40.D |  0ffe80  |   120    |  100006  |   128    | - [128       0f7008]

-----|----------|----------|----------|----------|------------------------

| positive | negative | positive | negative |

-----|----------|----------|----------|----------|------------------------

41.A |  400000  |   126    |  400000  |   126    | + [127 	400000]

-----|----------|----------|----------|----------|------------------------

42.B |  000007  |   120    |  000001  |   123    | + [123       100002]

-----|----------|----------|----------|----------|------------------------

43.C |  000004  |   120    |  000008  |   123    | + [123       100008]

-----|----------|----------|----------|----------|------------------------

44.D |  000004  |   120    |  000009  |   123    | + [123       10000a]

-----|----------|----------|----------|----------|------------------------

| positive | negative | positive | positive |

-----|----------|----------|----------|----------|------------------------

45.A |  000000  |   126    |  600000  |   129    | +[129	700000]

-----|----------|----------|----------|----------|------------------------

46.B |  000048  |   139    |  000401  |   128    | + [139       001049]

-----|----------|----------|----------|----------|------------------------

47.C |  000048  |   139    |  000400  |   128    | + [139       001048]

-----|----------|----------|----------|----------|------------------------

48.D |  000049  |   139    |  000400  |   128    | + [139       00104a]

-----|----------|----------|----------|----------|------------------------

| positive | positive | negative | negative |

-----|----------|----------|----------|----------|------------------------

49.A |  200000  |   129    |  000000  |   125    | + [129 	180000]

-----|----------|----------|----------|----------|------------------------

50.B |  01000d  |   128    |  7fff5f  |   120    | + [128       00000e]

-----|----------|----------|----------|----------|------------------------

51.C |  01000e  |   128    |  7fffe0  |   121    | + [127       7e001c]

-----|----------|----------|----------|----------|------------------------

52.D |  01000f  |   128    |  7fffe0  |   121    | + [127       7e001e]

-----|----------|----------|----------|----------|------------------------

| positive | positive | negative | positive |

-----|----------|----------|----------|----------|------------------------

53.A |  100000  |   130    |  300000  |   130    | - [128	000000]

-----|----------|----------|----------|----------|------------------------

54.B |  7ffff7  |   149    |  100021  |   153    | - [153       000022]

-----|----------|----------|----------|----------|------------------------

55.C |  7ffff8  |   149    |  100022  |   153    | - [153       000022]

-----|----------|----------|----------|----------|------------------------

56.D |  7ffff8  |   149    |  100023  |   153    | - [153       000024]

-----|----------|----------|----------|----------|------------------------

| positive | positive | positive | negative |

-----|----------|----------|----------|----------|------------------------

57.A |  580000  |   130    |  000000  |   126    | + [130	600000]

-----|----------|----------|----------|----------|------------------------

58.B |  000103  |   141    |  008001  |   125    | + [141       000184]

-----|----------|----------|----------|----------|------------------------

59.C |  000106  |   141    |  008000  |   125    | + [141       000186]

-----|----------|----------|----------|----------|------------------------

60.D |  000107  |   141    |  008000  |   125    | + [141       000188]

-----|----------|----------|----------|----------|------------------------

| positive | positive | positive | positive |

-----|----------|----------|----------|----------|------------------------

61.A |  180000  |   131    |  400000  |   129    | + [131	480000]

-----|----------|----------|----------|----------|------------------------

62.B |  00020f  |   130    |  002003  |   128    | + [130       200a10]

-----|----------|----------|----------|----------|------------------------

63.C |  00020e  |   130    |  002002  |   128    | + [130       200a0e]

-----|----------|----------|----------|----------|------------------------

64.D |  00020f  |   130    |  002002  |   128    | + [130       200a10]

_____|__________|__________|__________|__________|________________________

NOMINAL SUBTRACTION OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

| negative | negative | negative | negative |

-----|----------|----------|----------|----------|------------------------

65.A |  000000  |   125    |  000000  |   126    | + [125	000000]

-----|----------|----------|----------|----------|------------------------

66.B |	7ffff3  |   122    |  000001  |   126    | + [125       600004]

-----|----------|----------|----------|----------|------------------------

67.C |  7ffffc  |   122    |  000002  |   126    | + [125       600004]

-----|----------|----------|----------|----------|------------------------

68.D |  7ffff4  |   122    |  000003  |   126    | + [125       600008]

-----|----------|----------|----------|----------|------------------------

| negative | negative | negative | positive |

-----|----------|----------|----------|----------|------------------------

69.A |  400000  |   126    |  000000  |   128    | + [127	200000]

-----|----------|----------|----------|----------|------------------------

70.B |  7fffe2  |   126    |  100002  |   129    | + [128       60000c]

-----|----------|----------|----------|----------|------------------------

71.C |  7fffe4  |   126    |  200003  |   129    | + [129       000006]

-----|----------|----------|----------|----------|------------------------

72.D |  7fffe4  |   126    |  200004  |   129    | + [129       000008]

-----|----------|----------|----------|----------|------------------------

| negative | negative | positive | negative |

-----|----------|----------|----------|----------|------------------------

73.A |  000000  |   126    |  000000  |   124    | + [126	200000]

-----|----------|----------|----------|----------|------------------------

74.B |  000005  |   123    |  000000  |   126    | - [126       100001]

-----|----------|----------|----------|----------|------------------------

75.C |  000004  |   123    |  000000  |   126    | - [126       100000]

-----|----------|----------|----------|----------|------------------------

76.D |  000004  |   123    |  000001  |   126    | - [126       100002]

-----|----------|----------|----------|----------|------------------------

| negative | negative | positive | positive |

-----|----------|----------|----------|----------|------------------------

77.A |  200000  |   126	   |  000000  |   127    | - [127 	500000]

-----|----------|----------|----------|----------|------------------------

78.B |  000009  |   126    |  000001  |   130    | - [130       080002]

-----|----------|----------|----------|----------|------------------------

79.C |  000008  |   126    |  000004  |   130    | - [130       080004]

-----|----------|----------|----------|----------|------------------------

80.D |  000008  |   126    |  000005  |   130    | - [130       080006]

-----|----------|----------|----------|----------|------------------------

| negative | positive | negative | negative |

-----|----------|----------|----------|----------|------------------------

81.A |  400000  |   127	   |  000000  |   126    | - [127    	000000]

-----|----------|----------|----------|----------|------------------------

82.B |  100001  |   129    |  7fff70  |   121    | - [129       0f0002]

-----|----------|----------|----------|----------|------------------------

83.C |	100002  |   129    |  7fff80  |   121    | - [129       0f0002]

-----|----------|----------|----------|----------|------------------------

84.D |  100003  |   129    |  7fff80  |   121    | - [129       0f0004]

-----|----------|----------|----------|----------|------------------------

| negative | positive | negative | positive |

-----|----------|----------|----------|----------|------------------------

85.A |  300000  |   128    |  300000  |   128    | + [0		000000]

-----|----------|----------|----------|----------|------------------------

86.B |  020003  |   138    |  7fff21  |   130    | - [138       010004]

-----|----------|----------|----------|----------|------------------------

87.C |  020006  |   138    |  7fff80  |   130    | - [138       010006]

-----|----------|----------|----------|----------|------------------------

88.D |  020007  |   138    |  7fff80  |   130    | - [138       010008]

-----|----------|----------|----------|----------|------------------------

| negative | positive | positive | negative |

-----|----------|----------|----------|----------|------------------------

89.A | 	400000  |   128    |  000000  |   126    | - [128 	600000]

-----|----------|----------|----------|----------|------------------------

90.B |  00000b  |   128    |  000034  |   123    | - [128       04000d]

-----|----------|----------|----------|----------|------------------------

91.C |  00000b  |   128    |  000030  |   123    | - [128       04000c]

-----|----------|----------|----------|----------|------------------------

92.D |  00000c  |   128    |  000030  |   123    | - [128       04000e]

-----|----------|----------|----------|----------|------------------------

| negative | positive | positive | positive |

-----|----------|----------|----------|----------|------------------------

93.A |  000000  |   129    |  000000  |   129    | - [130 	000000]

-----|----------|----------|----------|----------|------------------------

94.B |  00000b  |   128    |  000001  |   132    | - [132       080002]

-----|----------|----------|----------|----------|------------------------

95.C |  000008  |   128    |  000006  |   132    | - [132       080006]

-----|----------|----------|----------|----------|------------------------

96.D |  000008  |   128    |  000007  |   132    | - [132       080008]

-----|----------|----------|----------|----------|------------------------

| positive | negative | negative | negative |

-----|----------|----------|----------|----------|------------------------

97.A |  000000  |   126    |  000000  |   125    | + [126	400000]

-----|----------|----------|----------|----------|------------------------

98.B |  000007  |   120    |  000001  |   123    | + [123       100002]

-----|----------|----------|----------|----------|------------------------

99.C |  000004  |   120    |  000008  |   123    | + [123       100008]

-----|----------|----------|----------|----------|------------------------

100.D|  000004  |   120    |  000009  |   123    | + [123       10000a]

-----|----------|----------|----------|----------|------------------------

| positive | negative | negative | positive |

-----|----------|----------|----------|----------|------------------------

101.A|  400000  |   126    |  000000  |   127    | + [127	600000]

-----|----------|----------|----------|----------|------------------------

102.B|	000048  |   139    |  000401  |   128    | + [139       001049]

-----|----------|----------|----------|----------|------------------------

103.C|  000048  |   139    |  000400  |   128    | + [139       001048]

-----|----------|----------|----------|----------|------------------------

104.D|  000049  |   139    |  000400  |   128    | + [139       00104a]

-----|----------|----------|----------|----------|------------------------

| positive | negative | positive | negative |

-----|----------|----------|----------|----------|------------------------

105.A|  400000  |   126    |  400000  |   126    | + [0		000000]

-----|----------|----------|----------|----------|------------------------

106.B|  100020  |   110    |  7fffa6  |   106    | + [110       000026]

-----|----------|----------|----------|----------|------------------------

107.C|  100021  |   110    |  7fffa8  |   106    | + [110       000026]

-----|----------|----------|----------|----------|------------------------

108.D|  100022  |   110    |  7fffa8  |   106    | + [110       000028]

-----|----------|----------|----------|----------|------------------------

| positive | negative | positive | positive |

-----|----------|----------|----------|----------|------------------------

109.A|  000000  |   126    |  600000  |   129    | - [129 	500000]

-----|----------|----------|----------|----------|------------------------

110.B|  7fff71  |   120    |  010005  |   128    | - [128       000006]

-----|----------|----------|----------|----------|------------------------

111.C|  0ffe80  |   120    |  100005  |   128    | - [128       0f7006]

-----|----------|----------|----------|----------|------------------------

112.D|  0ffe80  |   120    |  100006  |   128    | - [128       0f7008]

-----|----------|----------|----------|----------|------------------------

| positive | positive | negative | negative |

-----|----------|----------|----------|----------|------------------------

113.A|  200000  |   129    |  000000  |   125    | + [129   	280000]

-----|----------|----------|----------|----------|------------------------

114.B|  000103  |   141    |  008001  |   125    | + [141       000184]

-----|----------|----------|----------|----------|------------------------

115.C|  000106  |   141    |  008000  |   125    | + [141       000186]

-----|----------|----------|----------|----------|------------------------

116.D|  000107  |   141    |  008000  |   125    | + [141       000188]

-----|----------|----------|----------|----------|------------------------

| positive | positive | negative | positive |

-----|----------|----------|----------|----------|------------------------

117.A|  100000  |   130    |  300000  |   130    | + [131	200000]

-----|----------|----------|----------|----------|------------------------

118.B|  00020f  |   130    |  002003  |   128    | + [130       200a10]

-----|----------|----------|----------|----------|------------------------

119.C|  00020e  |   130    |  002002  |   128    | + [130       200a0e]

-----|----------|----------|----------|----------|------------------------

120.D|  00020f  |   130    |  002002  |   128    | + [130       200a10]

-----|----------|----------|----------|----------|------------------------

| positive | positive | positive | negative |

-----|----------|----------|----------|----------|------------------------

121.A|  580000  |   130    |  000000  |   126    | + [130 	500000]

-----|----------|----------|----------|----------|------------------------

122.B|  01000d  |   128    |  7fff5f  |   120    | + [128       00000e]

-----|----------|----------|----------|----------|------------------------

123.C|  01000e  |   128    |  7fffe0  |   121    | + [127       7e001c]

-----|----------|----------|----------|----------|------------------------

124.D|  01000f  |   128    |  7fffe0  |   121    | + [127       7e001e]

-----|----------|----------|----------|----------|------------------------

| positive | positive | positive | positive |

-----|----------|----------|----------|----------|------------------------

125.A|  180000  |   131    |  400000  |   129    | + [130 	500000]

-----|----------|----------|----------|----------|------------------------

126.B|  7ffff7  |   149    |  100021  |   153    | - [153       000022]

-----|----------|----------|----------|----------|------------------------

127.C|  7ffff8  |   149    |  100022  |   153    | - [153       000022]

-----|----------|----------|----------|----------|------------------------

128.D|  7ffff8  |   149    |  100023  |   153    | - [153       000024]

_____|__________|__________|__________|__________|________________________

IDLE OPERATION

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

129 |   ----   |   ----   |   ----   |   ----   |      No operation

_____|__________|__________|__________|__________|________________________

BOUNDARY (POS_ZERO) ADD OPERATIONS

(Positive zero)

IEEE format for POS_ZERO ->  sign = 0,  exponent = 0,  mantissa = 0

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

130 | negative | negative | positive |   zero   | -0.375 + 0 = -0.375

-----|----------|----------|----------|----------|------------------------

131 | negative | positive | positive |   zero   | -2.625 + 0 = -2.625

-----|----------|----------|----------|----------|------------------------

132 | positive | negative | positive |   zero   | 0.8125 + 0 = 0.8125

-----|----------|----------|----------|----------|------------------------

133 | positive | positive | positive |   zero   | 7.5 + 0 = 7.5

-----|----------|----------|----------|----------|------------------------

134 | positive |   zero   | negative | negative | 0 + (-0.375) = -0.375

-----|----------|----------|----------|----------|------------------------

135 | positive |   zero   | negative | positive | 0 + (-2.625) = -2.625

-----|----------|----------|----------|----------|------------------------

136 | positive |   zero   | positive | negative | 0 + 0.8125 = 0.8125

-----|----------|----------|----------|----------|------------------------

137 | positive |   zero   | positive | positive | 0 + 7.5 = 7.5

-----|----------|----------|----------|----------|------------------------

138 | positive |   zero   | positive |   zero   | 0 + 0 = 0

-----|----------|----------|----------|----------|------------------------

139 | positive |   zero   | positive |   zero   | 0 + MIN_POS = MIN_POS

-----|----------|----------|----------|----------|------------------------

140 | positive |   zero   | negative |   zero   | 0 + MIN_NEG = MIN_NEG

-----|----------|----------|----------|----------|------------------------

141 | positive |   zero   | positive | 1111..10 | 0 + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|------------------------

142 | positive |   zero   | negative | 1111..10 | 0 + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|------------------------

143 | positive |   zero   | positive | all 1's  | 0 + POS_INF = POS_INF

-----|----------|----------|----------|----------|------------------------

144 | positive |   zero   | negative | all 1's  | 0 + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|------------------------

145 | positive |   zero   | positive | all 1's  | 0 + NAN = NAN

_____|__________|__________|__________|__________|________________________

BOUNDARY (POS_ZERO) SUBTRACT OPERATIONS

(Positive zero)

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

146 | negative | negative | positive |   zero   | -0.375 - 0 = -0.375

-----|----------|----------|----------|----------|------------------------

147 | negative | positive | positive |   zero   | -2.625 - 0 = -2.625

-----|----------|----------|----------|----------|------------------------

148 | positive | negative | positive |   zero   | 0.8125 - 0 = 0.8125

-----|----------|----------|----------|----------|------------------------

149 | positive | positive | positive |   zero   | 7.5 - 0 = 7.5

-----|----------|----------|----------|----------|------------------------

150 | positive |   zero   | negative | negative | 0 - (-0.375) = 0.375

-----|----------|----------|----------|----------|------------------------

151 | positive |   zero   | negative | positive | 0 - (-2.625) = 2.625

-----|----------|----------|----------|----------|------------------------

152 | positive |   zero   | positive | negative | 0 - 0.8125 = -0.8125

-----|----------|----------|----------|----------|------------------------

153 | positive |   zero   | positive | positive | 0 - 7.5 = -7.5

-----|----------|----------|----------|----------|------------------------

154 | positive |   zero   | positive |   zero   |    0 - 0 = 0

-----|----------|----------|----------|----------|------------------------

155 | positive |   zero   | positive |   zero   | 0 - MIN_POS = MIN_NEG

-----|----------|----------|----------|----------|------------------------

156 | positive |   zero   | negative |   zero   | 0 - MIN_NEG = MIN_POS

-----|----------|----------|----------|----------|------------------------

157 | positive |   zero   | positive | 1111..10 | 0 - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|------------------------

158 | positive |   zero   | negative | 1111..10 | 0 - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|------------------------

159 | positive |   zero   | positive | all 1's  | 0 - POS_INF = NEG_INF

-----|----------|----------|----------|----------|------------------------

160 | positive |   zero   | negative | all 1's  | 0 - NEG_INF = POS_INF

-----|----------|----------|----------|----------|------------------------

161 | positive |   zero   | positive | all 1's  | 0 - NAN = NAN

_____|__________|__________|__________|__________|________________________

BOUNDARY (NEG_ZERO) ADD OPERATIONS

IEEE format for NEG_ZERO ->  sign = 1,  exponent = 0,  mantissa = 0

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

162 | negative | negative | negative |   zero   | -0.375 + -0 = -0.375

-----|----------|----------|----------|----------|------------------------

163 | negative | positive | negative |   zero   | -2.625 + -0 = -2.625

-----|----------|----------|----------|----------|------------------------

164 | positive | negative | negative |   zero   | 0.8125 + -0 = 0.8125

-----|----------|----------|----------|----------|------------------------

165 | positive | positive | negative |   zero   | 7.5 + -0 = 7.5

-----|----------|----------|----------|----------|------------------------

166 | negative |   zero   | negative | negative | -0 + (-0.375) = -0.375

-----|----------|----------|----------|----------|------------------------

167 | negative |   zero   | negative | positive | -0 + (-2.625) = -2.625

-----|----------|----------|----------|----------|------------------------

168 | negative |   zero   | positive | negative | -0 + 0.8125 = 0.8125

-----|----------|----------|----------|----------|------------------------

169 | negative |   zero   | positive | positive | -0 + 7.5 = 7.5

-----|----------|----------|----------|----------|------------------------

170 | negative |   zero   | negative |   zero   | -0 + -0 = 0

-----|----------|----------|----------|----------|------------------------

171 | negative |   zero   | positive |   zero   | -0 + MIN_POS = MIN_POS

-----|----------|----------|----------|----------|------------------------

172 | negative |   zero   | negative |   zero   | -0 + MIN_NEG = MIN_NEG

-----|----------|----------|----------|----------|------------------------

173 | negative |   zero   | positive | 1111..10 | -0 + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|------------------------

174 | negative |   zero   | negative | 1111..10 | -0 + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|------------------------

175 | negative |   zero   | positive | all 1's  | -0 + POS_INF = POS_INF

-----|----------|----------|----------|----------|------------------------

176 | negative |   zero   | negative | all 1's  | -0 + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|------------------------

177 | negative |   zero   | positive | all 1's  | -0 + NAN = NAN

_____|__________|__________|__________|__________|________________________

BOUNDARY (NEG_ZERO) SUBTRACT OPERATIONS

(Negative zero)

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

178 | negative | negative | negative |   zero   | -0.375 - (-0) = -0.375

-----|----------|----------|----------|----------|------------------------

179 | negative | positive | negative |   zero   | -2.625 - (-0) = -2.625

-----|----------|----------|----------|----------|------------------------

180 | positive | negative | negative |   zero   | 0.8125 - (-0) = 0.8125

-----|----------|----------|----------|----------|------------------------

181 | positive | positive | negative |   zero   | 7.5 - (-0) = 7.5

-----|----------|----------|----------|----------|------------------------

182 | negative |   zero   | negative | negative | -0 - (-0.375) = 0.375

-----|----------|----------|----------|----------|------------------------

183 | negative |   zero   | negative | positive | -0 - (-2.625) = 2.625

-----|----------|----------|----------|----------|------------------------

184 | negative |   zero   | positive | negative | -0 - 0.8125 = -0.8125

-----|----------|----------|----------|----------|------------------------

185 | negative |   zero   | positive | positive | -0 - 7.5 = -7.5

-----|----------|----------|----------|----------|------------------------

186 | negative |   zero   | positive |   zero   |    -0 - 0 = 0

-----|----------|----------|----------|----------|------------------------

187 | negative |   zero   | positive |   zero   | -0 - MIN_POS = MIN_NEG

-----|----------|----------|----------|----------|------------------------

188 | negative |   zero   | negative |   zero   | -0 - MIN_NEG = MIN_POS

-----|----------|----------|----------|----------|------------------------

189 | negative |   zero   | positive | 1111..10 | -0 - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|------------------------

190 | negative |   zero   | negative | 1111..10 | -0 - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|------------------------

191 | negative |   zero   | positive | all 1's  | -0 - POS_INF = NEG_INF

-----|----------|----------|----------|----------|------------------------

192 | negative |   zero   | negative | all 1's  | -0 - NEG_INF = POS_INF

-----|----------|----------|----------|----------|------------------------

193 | negative |   zero   | positive | all 1's  | -0 - NAN = NAN

_____|__________|__________|__________|__________|________________________

BOUNDARY (MIN_POS) ADD OPERATIONS

(Minimal Positive Number)

IEEE format for MIN_POS ->  sign = 0,  exponent = 0,  mantissa = 000000..01

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

194 | negative | negative | positive |   zero   | -0.375 + MIN_POS = -0.375

-----|----------|----------|----------|----------|-----------------------------

195 | negative | positive | positive |   zero   | -2.625+MIN_POS = -2.625

-----|----------|----------|----------|----------|-----------------------------

196 | positive | negative | positive |   zero   | 0.8125 + MIN_POS = 0.8125

-----|----------|----------|----------|----------|-----------------------------

197 | positive | positive | positive |   zero   | 7.5 + MIN_POS = 7.5

-----|----------|----------|----------|----------|-----------------------------

198 | positive |   zero   | negative | negative | MIN_POS+(-0.375) = -0.375

-----|----------|----------|----------|----------|-----------------------------

199 | positive |   zero   | negative | positive | MIN_POS+(-2.625) = -2.625

-----|----------|----------|----------|----------|-----------------------------

200 | positive |   zero   | positive | negative | MIN_POS + 0.8125 = 0.8125

-----|----------|----------|----------|----------|-----------------------------

201 | positive |   zero   | positive | positive | MIN_POS + 7.5 = 7.5

-----|----------|----------|----------|----------|-----------------------------

202 | positive |   zero   | positive |   zero   | MIN_POS + 0 = MIN_POS

-----|----------|----------|----------|----------|-----------------------------

203 | positive |   zero   | positive |   zero   | MIN_POS+MIN_POS = 2*MIN_POS

-----|----------|----------|----------|----------|-----------------------------

204 | positive |   zero   | negative |   zero   | MIN_POS + MIN_NEG = 0

-----|----------|----------|----------|----------|-----------------------------

205 | positive |   zero   | positive | 1111..10 | MIN_POS + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

206 | positive |   zero   | negative | 1111..10 | MIN_POS + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

207 | positive |   zero   | positive | all 1's  | MIN_POS + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

208 | positive |   zero   | negative | all 1's  | MIN_POS + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

209 | positive |   zero   | positive | all 1's  | MIN_POS + NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (MIN_POS) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

210 | negative | negative |   zero   |   zero   | -0.375 - MIN_POS = -0.375

-----|----------|----------|----------|----------|----------------------------

211 | negative | positive | positive |   zero   | -2.625 - MIN_POS = -2.625

-----|----------|----------|----------|----------|----------------------------

212 | positive | negative | positive |   zero   | 0.8125-MIN_POS = 0.8125

-----|----------|----------|----------|----------|----------------------------

213 | positive | positive | positive |   zero   | 7.5 - MIN_POS = 7.5

-----|----------|----------|----------|----------|----------------------------

214 | positive |   zero   | negative | negative | MIN_POS - (-0.375) = 0.375

-----|----------|----------|----------|----------|----------------------------

215 | positive |   zero   | negative | positive | MIN_POS - (-2.625) = 2.625

-----|----------|----------|----------|----------|----------------------------

216 | positive |   zero   | positive | negative | MIN_POS-0.8125 = -0.8125

-----|----------|----------|----------|----------|----------------------------

217 | positive |   zero   | positive | positive | MIN_POS - 7.5 = -7.5

-----|----------|----------|----------|----------|----------------------------

218 | positive |   zero   | positive |   zero   | MIN_POS - 0 = MIN_POS

-----|----------|----------|----------|----------|----------------------------

219 | positive |   zero   | positive |   zero   | MIN_POS - MIN_POS = 0

-----|----------|----------|----------|----------|----------------------------

220 | positive |   zero   | negative |   zero   | MIN_POS-MIN_NEG = 2*MIN_POS

-----|----------|----------|----------|----------|----------------------------

221 | positive |   zero   | positive | 1111..10 | MIN_POS - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

222 | positive |   zero   | negative | 1111..10 | MIN_POS - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

223 | positive |   zero   | positive | all 1's  | MIN_POS - POS_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

224 | positive |   zero   | negative | all 1's  | MIN_POS - NEG_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

225 | positive |   zero   | positive | all 1's  | MIN_POS - NAN = NAN

_____|__________|__________|__________|__________|____________________________

BOUNDARY (MIN_NEG) ADD OPERATIONS

(Minimum negative number)

IEEE format for MIN_NEG ->  sign = 1,  exponent = 0,  mantissa = 000000..01

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

226 | negative | negative | negative |   zero   | -0.375 + MIN_NEG = -0.375

-----|----------|----------|----------|----------|-----------------------------

227 | negative | positive | negative |   zero   | -2.625+MIN_NEG = -2.625

-----|----------|----------|----------|----------|-----------------------------

228 | positive | negative | negative |   zero   | 0.8125 + MIN_NEG = 0.8125

-----|----------|----------|----------|----------|-----------------------------

229 | positive | positive | negative |   zero   | 7.5 + MIN_NEG = 7.5

-----|----------|----------|----------|----------|-----------------------------

230 | negative |   zero   | negative | negative | MIN_NEG + (-0.375) = -0.375

-----|----------|----------|----------|----------|-----------------------------

231 | negative |   zero   | negative | positive | MIN_NEG + (-2.625) = -2.625

-----|----------|----------|----------|----------|-----------------------------

232 | negative |   zero   | positive | negative | MIN_NEG + 0.8125 = 0.8125

-----|----------|----------|----------|----------|-----------------------------

233 | negative |   zero   | positive | positive | MIN_NEG + 7.5 = 7.5

-----|----------|----------|----------|----------|-----------------------------

234 | negative |   zero   | positive |   zero   | MIN_NEG + 0 = MIN_NEG

-----|----------|----------|----------|----------|-----------------------------

235 | negative |   zero   | positive |   zero   | MIN_NEG + MIN_POS = 0

-----|----------|----------|----------|----------|-----------------------------

236 | negative |   zero   | negative |   zero   | MIN_NEG+MIN_NEG = 2*MIN_NEG

-----|----------|----------|----------|----------|-----------------------------

237 | negative |   zero   | positive | 1111..10 | MIN_NEG + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

238 | negative |   zero   | negative | 1111..10 | MIN_NEG + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

239 | negative |   zero   | positive | all 1's  | MIN_POS + POS_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

240 | negative |   zero   | negative | all 1's  | MIN_POS + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

241 | negative |   zero   | positive | all 1's  | MIN_POS + NAN = NAN

_____|__________|__________|__________|__________|____________________________

BOUNDARY (MIN_NEG) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

242 | negative | negative | negative |   zero   | -0.375-MIN_NEG = -0.375

-----|----------|----------|----------|----------|----------------------------

243 | negative | positive | negative |   zero   | -2.625-MIN_NEG = -2.625

-----|----------|----------|----------|----------|----------------------------

244 | positive | negative | negative |   zero   | 0.8125-MIN_NEG = 0.8125

-----|----------|----------|----------|----------|----------------------------

245 | positive | positive | negative |   zero   | 7.5 - MIN_NEG = 7.5

-----|----------|----------|----------|----------|----------------------------

246 | negative |   zero   | negative | negative | MIN_NEG-(-0.375) = 0.375

-----|----------|----------|----------|----------|----------------------------

247 | negative |   zero   | negative | positive | MIN_NEG-(-2.625) = 2.65

-----|----------|----------|----------|----------|----------------------------

248 | negative |   zero   | positive | negative | MIN_NEG - 0.8125 = -0.8125

-----|----------|----------|----------|----------|----------------------------

249 | negative |   zero   | positive | positive | MIN_NEG - 7.5 = -7.5

-----|----------|----------|----------|----------|----------------------------

250 | negative |   zero   | positive |   zero   | MIN_NEG - 0 = MIN_NEG

-----|----------|----------|----------|----------|----------------------------

251 | negative |   zero   | positive |   zero   | MIN_NEG-MIN_POS = 2*MIN_NEG

-----|----------|----------|----------|----------|----------------------------

252 | negative |   zero   | negative |   zero   | MIN_NEG-MIN_NEG = 0

-----|----------|----------|----------|----------|----------------------------

253 | negative |   zero   | positive | 1111..10 | MIN_NEG - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

254 | negative |   zero   | negative | 1111..10 | MIN_NEG - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

255 | negative |   zero   | positive | all 1's  | MIN_NEG - POS_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

256 | negative |   zero   | negative | all 1's  | MIN_NEG - NEG_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

257 | negative |   zero   | positive | all 1's  | MIN_NEG - NAN = NAN

_____|__________|__________|__________|__________|____________________________

BOUNDARY (MAX_POS) ADD OPERATIONS

(Maximum positive number)

IEEE format for MAX_POS ->  sign = 0,  exponent = 254,  mantissa = 11111..11

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

258 | negative | negative | positive | 1111..10 | -0.375 + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

259 | negative | positive | positive | 1111..10 | -2.625 + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

260 | positive | negative | positive | 1111..10 | 0.8125 + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

261 | positive | positive | positive | 1111..10 | 7.5 + MAX_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

262 | positive | 1111..10 | negative | negative | MAX_POS + (-0.375) = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

263 | positive | 1111..10 | negative | positive | MAX_POS + (-2.625) = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

264 | positive | 1111..10 | positive | negative | MAX_POS + 0.8125 = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

265 | positive | 1111..10 | positive | positive | MAX_POS + 7.5 = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

266 | positive | 1111..10 | positive |   zero   | MAX_POS + 0 = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

267 | positive | 1111..10 | positive |   zero   | MAX_POS + MIN_POS = MAX_POS

-----|----------|----------|----------|----------|-----------------------------

268 | positive | 1111..10 | negative |   zero   | MAX_POS + MIN_NEG = 0

-----|----------|----------|----------|----------|-----------------------------

269 | positive | 1111..10 | positive | 1111..10 | MAX_POS + MAX_POS = POS_INF

-----|----------|----------|----------|----------|-----------------------------

270 | positive | 1111..10 | negative | 1111..10 | MAX_POS + MAX_NEG = 0

-----|----------|----------|----------|----------|-----------------------------

271 | positive | 1111..10 | positive | all 1's  | MAX_POS + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

272 | positive | 1111..10 | negative | all 1's  | MAX_POS + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

273 | positive | 1111..10 | positive | all 1's  | MAX_POS + NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (MAX_POS) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

274 | negative | negative | positive | 1111..10 | -0.375 - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

275 | negative | positive | positive | 1111..10 | -2.625 - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

276 | positive | negative | positive | 1111..10 | 0.8125 - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

277 | positive | positive | positive | 1111..10 | 7.5 - MAX_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

278 | positive | 1111..10 | negative | negative | MAX_POS-(-0.375) = MAX_POS

-----|----------|----------|----------|----------|----------------------------

279 | positive | 1111..10 | negative | positive | MAX_POS-(-2.625) = MAX_POS

-----|----------|----------|----------|----------|----------------------------

280 | positive | 1111..10 | positive | negative | MAX_POS - 0.8125 = MAX_POS

-----|----------|----------|----------|----------|----------------------------

281 | positive | 1111..10 | positive | positive | MAX_POS - 7.5 = MAX_POS

-----|----------|----------|----------|----------|----------------------------

282 | positive | 1111..10 | positive |   zero   | MAX_POS - 0 = MAX_POS

-----|----------|----------|----------|----------|----------------------------

283 | positive | 1111..10 | positive |   zero   | MAX_POS - MIN_POS = MAX_POS

-----|----------|----------|----------|----------|----------------------------

284 | positive | 1111..10 | negative |   zero   | MAX_POS - MIN_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

285 | positive | 1111..10 | positive | 1111..10 | MAX_POS - MAX_POS = 0

-----|----------|----------|----------|----------|----------------------------

286 | positive | 1111..10 | negative | 1111..10 | MAX_POS - MAX_NEG = POS_INF

-----|----------|----------|----------|----------|-----------------------------

287 | positive | 1111..10 | positive | all 1's  | MAX_POS - POS_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

288 | positive | 1111..10 | negative | all 1's  | MAX_POS - NEG_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

289 | positive | 1111..10 | positive | all 1's  | MAX_POS - NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (MAX_NEG) ADD OPERATIONS

(Maximum negative number)

IEEE format for MAX_POS ->  sign = 1,  exponent = 254,  mantissa = 11111..11

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

290 | negative | negative | negative | 1111..10 | -0.375 + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

291 | negative | positive | negative | 1111..10 | -2.625 + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

292 | positive | negative | negative | 1111..10 | 0.8125 + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

293 | positive | positive | negative | 1111..10 | 7.5 + MAX_NEG = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

294 | negative | 1111..10 | negative | negative | MAX_NEG + (-0.375) = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

295 | negative | 1111..10 | negative | positive | MAX_NEG + (-2.625) = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

296 | negative | 1111..10 | positive | negative | MAX_NEG + 0.8125 = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

297 | negative | 1111..10 | positive | positive | MAX_NEG + 7.5 = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

298 | negative | 1111..10 | positive |   zero   | MAX_NEG + 0 = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

299 | negative | 1111..10 | positive |   zero   | MAX_NEG + MIN_POS = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

300 | negative | 1111..10 | negative |   zero   | MAX_NEG + MIN_NEG = MAX_NEG

-----|----------|----------|----------|----------|-----------------------------

301 | negative | 1111..10 | positive | 1111..10 | MAX_NEG + MAX_POS = 0

-----|----------|----------|----------|----------|-----------------------------

302 | negative | 1111..10 | negative | 1111..10 | MAX_NEG + MAX_NEG = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

303 | negative | 1111..10 | positive | all 1's  | MAX_NEG + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

304 | negative | 1111..10 | negative | all 1's  | MAX_NEG + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

305 | negative | 1111..10 | positive | all 1's  | MAX_NEG + NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (MAX_NEG) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

306 | negative | negative | negative | 1111..10 | -0.375 - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

307 | negative | positive | negative | 1111..10 | -2.625 - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

308 | positive | negative | negative | 1111..10 | 0.8125 - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

309 | positive | positive | negative | 1111..10 | 7.5 - MAX_NEG = MAX_POS

-----|----------|----------|----------|----------|----------------------------

310 | negative | 1111..10 | negative | negative | MAX_NEG-(-0.375) = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

311 | negative | 1111..10 | negative | positive | MAX_NEG-(-2.625) = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

312 | negative | 1111..10 | positive | negative | MAX_NEG - 0.8125 = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

313 | negative | 1111..10 | positive | positive | MAX_NEG - 7.5 = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

314 | negative | 1111..10 | positive | 1111..10 | MAX_NEG - 0 = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

315 | negative | 1111..10 | positive | 1111..10 | MAX_NEG - MIN_POS = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

316 | negative | 1111..10 | negative | 1111..10 | MAX_NEG-MIN_NEG = MAX_NEG

-----|----------|----------|----------|----------|----------------------------

317 | negative | 1111..10 | positive | 1111..10 | MAX_NEG - MAX_POS = NEG_INF

-----|----------|----------|----------|----------|----------------------------

318 | negative | 1111..10 | negative | 1111..10 | MAX_NEG - MAX_NEG = 0

-----|----------|----------|----------|----------|-----------------------------

319 | negative | 1111..10 | positive | all 1's  | MAX_NEG - POS_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

320 | negative | 1111..10 | negative | all 1's  | MAX_NEG - NEG_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

321 | negative | 1111..10 | positive | all 1's  | MAX_NEG - NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (POS_INF) ADD OPERATIONS

(Positive infinity)

IEEE format for POS_INF ->  sign = 0,  exponent = 255,  mantissa = all 0's

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

322 | negative | negative | positive | all 1's  | -0.375 + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

323 | negative | positive | positive | all 1's  | -2.625 + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

324 | positive | negative | positive | all 1's  | 0.8125 + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

325 | positive | positive | positive | all 1's  | 7.5 + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

326 | positive | all 1's  | negative | negative | POS_INF + (-0.375) = POS_INF

-----|----------|----------|----------|----------|-----------------------------

327 | positive | all 1's  | negative | positive | POS_INF + (-2.625) = POS_INF

-----|----------|----------|----------|----------|-----------------------------

328 | positive | all 1's  | positive | negative | POS_INF + 0.8125 = POS_INF

-----|----------|----------|----------|----------|-----------------------------

329 | positive | all 1's  | positive | positive | POS_INF + 7.5 = POS_INF

-----|----------|----------|----------|----------|-----------------------------

330 | positive | all 1's  | positive |   zero   | POS_INF + 0 = POS_INF

-----|----------|----------|----------|----------|-----------------------------

331 | positive | all 1's  | positive |   zero   | POS_INF + MIN_POS = POS_INF

-----|----------|----------|----------|----------|-----------------------------

332 | positive | all 1's  | negative |   zero   | POS_INF + MIN_NEG = POS_INF

-----|----------|----------|----------|----------|-----------------------------

333 | positive | all 1's  | positive | 1111.10  | POS_INF + MAX_POS = POS_INF

-----|----------|----------|----------|----------|-----------------------------

334 | positive | all 1's  | negative | 1111..10 | POS_INF + MAX_NEG = POS_INF

-----|----------|----------|----------|----------|-----------------------------

335 | positive | all 1's  | positive | all 1's  | POS_INF + POS_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

336 | positive | all 1's  | negative | all 1's  | POS_INF + NEG_INF = 0

-----|----------|----------|----------|----------|-----------------------------

337 | positive | all 1's  | positive | all 1's  | POS_INF + NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (POS_INF) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

338 | negative | negative | positive | all 1's  | -0.375 - POS_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

339 | negative | positive | positive | all 1's  | -2.625 - POS_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

340 | positive | negative | positive | all 1's  | 0.8125 - POS_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

341 | positive | positive | positive | all 1's  | 7.5 - POS_INF = NEG_INF

-----|----------|----------|----------|----------|----------------------------

342 | positive | all 1's  | negative | negative | POS_INF-(-0.375) = POS_INF

-----|----------|----------|----------|----------|----------------------------

343 | positive | all 1's  | negative | positive | POS_INF-(-2.625) = POS_INF

-----|----------|----------|----------|----------|----------------------------

344 | positive | all 1's  | positive | negative | POS_INF - 0.8125 = POS_INF

-----|----------|----------|----------|----------|----------------------------

345 | positive | all 1's  | positive | positive | POS_INF - 7.5 = POS_INF

-----|----------|----------|----------|----------|----------------------------

346 | positive | all 1's  | positive |   zero   | POS_INF - 0 = POS_INF

-----|----------|----------|----------|----------|----------------------------

347 | positive | all 1's  | positive |   zero   | POS_INF - MIN_POS = POS_INF

-----|----------|----------|----------|----------|----------------------------

348 | positive | all 1's  | negative |   zero   | POS_INF - MIN_NEG = POS_INF

-----|----------|----------|----------|----------|----------------------------

349 | positive | all 1's  | positive | 1111..10 | POS_INF - MAX_POS = POS_INF

-----|----------|----------|----------|----------|----------------------------

350 | positive | all 1's  | negative | all 1's  | POS_INF - MAX_NEG = POS_INF

-----|----------|----------|----------|----------|-----------------------------

351 | positive | all 1's  | positive | all 1's  | POS_INF - POS_INF = 0

-----|----------|----------|----------|----------|-----------------------------

352 | positive | all 1's  | negative | all 1's  | POS_INF - NEG_INF = POS_INF

-----|----------|----------|----------|----------|-----------------------------

353 | positive | all 1's  | positive | all 1's  | POS_INF - NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (NEG_INF) ADD OPERATIONS

(Negative infinity)

IEEE format for NEG_INF ->  sign = 1,  exponent = 255,  mantissa = all 0's

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

354 | negative | negative | negative | all 1's  | -0.375 + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

355 | negative | positive | negative | all 1's  | -2.625 + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

356 | positive | negative | negative | all 1's  | 0.8125 + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

357 | positive | positive | negative | all 1's  | 7.5 + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

358 | negative | all 1's  | negative | negative | NEG_INF + (-0.375) = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

359 | negative | all 1's  | negative | positive | NEG_INF + (-2.625) = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

360 | negative | all 1's  | positive | negative | NEG_INF + 0.8125 = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

361 | negative | all 1's  | positive | positive | NEG_INF + 7.5 = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

362 | negative | all 1's  | positive |   zero   | NEG_INF + 0 = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

363 | negative | all 1's  | positive |   zero   | NEG_INF + MIN_POS = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

364 | negative | all 1's  | negative |   zero   | NEG_INF + MIN_NEG = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

365 | negative | all 1's  | positive | 1111.10  | NEG_INF + MAX_POS = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

366 | negative | all 1's  | negative | 1111..10 | NEG_INF + MAX_NEG = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

367 | negative | all 1's  | positive | all 1's  | NEG_INF + POS_INF = 0

-----|----------|----------|----------|----------|-----------------------------

368 | negative | all 1's  | negative | all 1's  | NEG_INF + NEG_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

369 | negative | all 1's  | positive | all 1's  | NEG_INF + NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (NEG_INF) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

370 | negative | negative | negative | all 1's  | -0.375 - NEG_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

371 | negative | positive | negative | all 1's  | -2.625 - NEG_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

372 | positive | negative | negative | all 1's  | 0.8125 - NEG_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

373 | positive | positive | negative | all 1's  | 7.5 - NEG_INF = POS_INF

-----|----------|----------|----------|----------|----------------------------

374 | negative | all 1's  | negative | negative | NEG_INF-(-0.375) = NEG_INF

-----|----------|----------|----------|----------|----------------------------

375 | negative | all 1's  | negative | positive | NEG_INF-(-2.625) = NEG_INF

-----|----------|----------|----------|----------|----------------------------

376 | negative | all 1's  | positive | negative | NEG_INF - 0.8125 = NEG_INF

-----|----------|----------|----------|----------|----------------------------

377 | negative | all 1's  | positive | positive | NEG_INF - 7.5 = NEG_INF

-----|----------|----------|----------|----------|----------------------------

378 | negative | all 1's  | positive |   zero   | NEG_INF - 0 = NEG_INF

-----|----------|----------|----------|----------|----------------------------

379 | negative | all 1's  | positive |   zero   | NEG_INF - MIN_POS = NEG_INF

-----|----------|----------|----------|----------|----------------------------

380 | negative | all 1's  | negative |   zero   | NEG_INF - MIN_NEG = NEG_INF

-----|----------|----------|----------|----------|----------------------------

381 | negative | all 1's  | positive | 1111..10 | NEG_INF - MAX_POS = NEG_INF

-----|----------|----------|----------|----------|----------------------------

382 | negative | all 1's  | negative | 1111..10 | NEG_INF - MAX_NEG = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

383 | negative | all 1's  | positive | all 1's  | NEG_INF - POS_INF = NEG_INF

-----|----------|----------|----------|----------|-----------------------------

384 | negative | all 1's  | negative | all 1's  | NEG_INF - NEG_INF = 0

-----|----------|----------|----------|----------|-----------------------------

385 | negative | all 1's  | positive | all 1's  | NEG_INF - NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (NAN) ADD OPERATIONS

(Not a number)

IEEE format for NAN ->  sign = 0 or 1,  exponent = 255,  mantissa = not all 0's

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

386 | negative | negative | positive | all 1's  | -0.375 + NAN = NAN

-----|----------|----------|----------|----------|-----------------------------

387 | negative | positive | positive | all 1's  | -2.625 + NAN = NAN

-----|----------|----------|----------|----------|-----------------------------

388 | positive | negative | positive | all 1's  | 0.8125 + NAN = NAN

-----|----------|----------|----------|----------|-----------------------------

389 | positive | positive | positive | all 1's  | 7.5 + NAN = NAN

-----|----------|----------|----------|----------|-----------------------------

390 | positive | all 1's  | negative | negative | NAN + (-0.375) = NAN

-----|----------|----------|----------|----------|-----------------------------

391 | positive | all 1's  | negative | positive | NAN + (-2.625) = NAN

-----|----------|----------|----------|----------|-----------------------------

392 | positive | all 1's  | positive | negative | NAN + 0.8125 = NAN

-----|----------|----------|----------|----------|-----------------------------

393 | positive | all 1's  | positive | positive | NAN + 7.5 = NAN

-----|----------|----------|----------|----------|-----------------------------

394 | positive | all 1's  | positive |   zero   | NAN + 0 = NAN

-----|----------|----------|----------|----------|-----------------------------

395 | positive | all 1's  | positive |   zero   | NAN + MIN_POS = NAN

-----|----------|----------|----------|----------|-----------------------------

396 | positive | all 1's  | negative |   zero   | NAN + MIN_NEG = NAN

-----|----------|----------|----------|----------|-----------------------------

397 | positive | all 1's  | positive | 1111.10  | NAN + MAX_POS = NAN

-----|----------|----------|----------|----------|-----------------------------

398 | positive | all 1's  | negative | 1111..10 | NAN + MAX_NEG = NAN

-----|----------|----------|----------|----------|-----------------------------

399 | positive | all 1's  | positive | all 1's  | NAN + POS_INF = NAN

-----|----------|----------|----------|----------|-----------------------------

400 | positive | all 1's  | negative | all 1's  | NAN + NEG_INF = NAN

-----|----------|----------|----------|----------|-----------------------------

401 | positive | all 1's  | positive | all 1's  | NAN + NAN = NAN

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (NAN) SUBTRACT OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

402 | negative | negative | positive | all 1's  | -0.375 - NAN = NAN

-----|----------|----------|----------|----------|----------------------------

403 | negative | positive | positive | all 1's  | -2.625 - NAN = NAN

-----|----------|----------|----------|----------|----------------------------

404 | positive | negative | positive | all 1's  | 0.8125 - NAN = NAN

-----|----------|----------|----------|----------|----------------------------

405 | positive | positive | positive | all 1's  | 7.5 - NAN = NAN

-----|----------|----------|----------|----------|----------------------------

406 | positive | all 1's  | negative | negative | NAN-(-0.375) = NAN

-----|----------|----------|----------|----------|----------------------------

407 | positive | all 1's  | negative | positive | NAN-(-2.625) = NAN

-----|----------|----------|----------|----------|----------------------------

408 | positive | all 1's  | positive | negative | NAN - 0.8125 = NAN

-----|----------|----------|----------|----------|----------------------------

409 | positive | all 1's  | positive | positive | NAN - 7.5 = NAN

-----|----------|----------|----------|----------|----------------------------

410 | positive | all 1's  | positive |   zero   | NAN - 0 = NAN

-----|----------|----------|----------|----------|----------------------------

411 | positive | all 1's  | positive |   zero   | NAN - MIN_POS = NAN

-----|----------|----------|----------|----------|----------------------------

412 | positive | all 1's  | negative |   zero   | NAN - MIN_NEG = NAN

-----|----------|----------|----------|----------|----------------------------

413 | positive | all 1's  | positive | 1111..10 | NAN - MAX_POS = NAN

-----|----------|----------|----------|----------|----------------------------

414 | positive | all 1's  | negative | all 1's  | NAN - MAX_NEG = NAN

-----|----------|----------|----------|----------|-----------------------------

415 | positive | all 1's  | positive | all 1's  | NAN - POs_INF = NAN

-----|----------|----------|----------|----------|-----------------------------

416 | positive | all 1's  | negative | all 1's  | NAN - NEG_INF = NAN

-----|----------|----------|----------|----------|-----------------------------

417 | positive | all 1's  | positive | all 1's  | NAN - NAN = NAN

_____|__________|__________|__________|__________|_____________________________