-------------------------------------------------------------------------------

--

-- Floating Point Multiplier Benchmark: Test Documentation

--

-- Source:  Patterson, David A., and Hennessy, John L.,  "Computer

--               architecture: a quantitative approach".  San Mateo, CA: Morgank

--               Kaufman Publishers, 1990,  Appendix A, p. 3-22

--

-- Author: Jesse Pan

--         Department of Electrical and Computer Engineering

--         University of California, Irvine, CA 92717

--

-- Acknowledgement: Special thanks to Dr. Tomas Lang's advice on this benchmark

--

-- Written on Mar 01, 1994

--

-------------------------------------------------------------------------------

TESTING THE Floating Point Multiplier PROCESS.

PORTS

clk		input

op1_sign	input

op1_exp		input

op1_mant	input

op2_sign	input

op2_exp		input

op2_mant	input

res_sign	output

res_exp		output

res_mant	output

operation	input

flags		output

Details about these ports are mentioned in the FP_mult.doc file.

OVERALL TESTING STRATEGY

In testing the Multiplier, the overall strategy is to test for typical

behaviors and exhaustive testing of boundary condition, performing

operations among the following significant operands.

In addition to nominal cases and modes of operations, extensive tests on

boundary cases (e.g. Not-a-Number, +/- Infinity, +/- Max and  +/-Min) were

done.

TESTING THE Multiplier

Now, we will chart the testing of the Floating Point Multiplier.

*************************************************************************

*								        *

* TEST VECTORS FOR OUTPUT PRODUCED FROM OPERATION PERFORMED ON OPERANDS *

*								        *

*************************************************************************

IEEE format for Nominal numbers ->

sign = 0 or 1,  0 <= exponent <= 254 ,  mantissa = any 23-bit vector

Note: if exponent = 0 and mantissa is all zeros, this is not a nominal number,

it is the special representation of zero.

The follow table shows the typical test vectors and the result of operations.

The MSB in the MANT field are octave; the remaining five digit are hexadecimal.

****************************************************************************

*    |  OPERAND 1   | OPERAND 2    |  RESULT      |                        *

*TEST|   |   | Hex  |   |   | Hex  |   |   | Hex  |        OPERATION       *

*  # |SGN|EXP| MANT |SGN|EXP| MANT |SGN|EXP| MANT |                        *

*----|---|---|------|---|---|------|---|---|------|------------------------*

*  1 | 1 |125|000000| 1 |126|000000| 1 |126|400000|-0.25 * (-0.5) = 0.125  *

*----|---|---|------|---|---|------|---|---|------|------------------------*

*  2 | 1 |126|400000| 0 |128|000000| 1 |128|300000|-0.75 * (-2.0) = 1.5    *

*----|---|---|------|---|---|------|---------------------------------------*

*  3 | 1 |126|000000| 0 |124|000000| 1 |125|400000|-0.5 * 0.125  = -0.0625 *

****************************************************************************

* The following table contents show the sign of the numbers and the sign of the

of the exponents after the bias are subtracted.

NOMINAL OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |  OPERATION RESULT

#  | SIGN/MANT| EXPONENT | SIGN/MANT| EXPONENT |

-----|----------|----------|----------|----------|------------------------

1   | -000000  |   125    | -000000  |   126    |   [000000	124]

-----|----------|----------|----------|----------|------------------------

2   | -400000  |   126    | -000000  |   128    |   [400000	127]

-----|----------|----------|----------|----------|------------------------

3   | -000000  |   126    |  000000  |   124    | - [000000	123]

-----|----------|----------|----------|----------|------------------------

4   | -200000  |   126    |  000000  |   127    | - [200000 	126]

-----|----------|----------|----------|----------|------------------------

5   | -400000  |   127    | -000000  |   126    |   [400000	126]

-----|----------|----------|----------|----------|------------------------

6   | -300000  |   128    | -300000  |   128    |   [720000	129]

-----|----------|----------|----------|----------|------------------------

7   | -400000  |   128    |  000000  |   126    | - [400000 	127]

-----|----------|----------|----------|----------|------------------------

8   | -000000  |   129    |  000000  |   129    | - [000000	131]

-----|----------|----------|----------|----------|------------------------

9   |  000000  |   126    | -000000  |   125    | - [000000	124]

-----|----------|----------|----------|----------|------------------------

10   |  400000  |   126    | -000000  |   127    | - [400000	126]

-----|----------|----------|----------|----------|------------------------

11   |  400000  |   126    |  400000  |   126    |   [100000	126]

-----|----------|----------|----------|----------|------------------------

12   |  000000  |   126    |  600000  |   129    |   [600000	128]

-----|----------|----------|----------|----------|------------------------

13   |  200000  |   129    | -000000  |   125    | - [200000	127]

-----|----------|----------|----------|----------|------------------------

14   |  100000  |   130    | -300000  |   130    | - [460000	133]

-----|----------|----------|----------|----------|------------------------

15   |  580000  |   130    |  000000  |   126    |   [580000	129]

-----|----------|----------|----------|----------|------------------------

16   |  180000  |   131    |  400000  |   129    |   [640008	133]

-----|----------|----------|----------|----------|------------------------

17   | -000000  |   125    | -000000  |   126    |   [000000	124]

-----|----------|----------|----------|----------|------------------------

18   | -400000  |   126    | -000000  |	  128    |   [400000	127]

-----|----------|----------|----------|----------|------------------------

19   | -000000  |   126    |  000000  |   124    | - [000000	123]

-----|----------|----------|----------|----------|------------------------

20   | -200000  |   126    |  000000  |   127    | - [200000	126]

-----|----------|----------|----------|----------|------------------------

21   | -400000  |   127    | -000000  |   126    |   [400000 	126]

-----|----------|----------|----------|----------|------------------------

22   | -300000  |   128    | -300000  |   128    |   [720000	129]

-----|----------|----------|----------|----------|------------------------

23   | -400000  |   128    |  000000  |   126    | - [400000 	127]

-----|----------|----------|----------|----------|------------------------

24   | -000000  |   129    |  000000  |   129    | - [000000	131]

-----|----------|----------|----------|----------|------------------------

25   |  000000  |   126    | -000000  |   125    | - [000000	124]

-----|----------|----------|----------|----------|------------------------

26   |  400000  |   126    | -000000  |   127    | - [400000	126]

-----|----------|----------|----------|----------|------------------------

27   |  400000  |   126    |  400000  |   126    | - [100000    126]

-----|----------|----------|----------|----------|------------------------

28   |  000000  |   126    |  600000  |   129    |   [600000	128]

-----|----------|----------|----------|----------|------------------------

29   |  200000  |   129    | -000000  |   125    | - [200000	127]

-----|----------|----------|----------|----------|------------------------

30   |  100000  |   130    | -300000  |   130    | - [460000 	133]

-----|----------|----------|----------|----------|------------------------

31   |  580000  |   130    |  000000  |   126    |   [580000	129]

-----|----------|----------|----------|----------|------------------------

32   |  180000  |   131    |  400000  |   129    |   [640000	133]

_____|__________|__________|__________|__________|________________________

IDLE OPERATION

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |   SIGN   | EXPONENT |   SIGN   | EXPONENT |

-----|----------|----------|----------|----------|------------------------

33  |   ----   |   ----   |   ----   |   ----   |      No operation

_____|__________|__________|__________|__________|________________________

BOUNDARY (POS_ZERO) OPERATIONS

(Positive zero)

IEEE format for POS_ZERO ->  sign = 0,  exponent = 0,  mantissa = 0

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|------------------------

34  | -400000  |   125    |  000000  |     0    | -[000000	0]

-----|----------|----------|----------|----------|------------------------

35  | -280000  |   128    |  000000  |     0    | -[000000	0]

-----|----------|----------|----------|----------|------------------------

36  |  500000  |   126    |  000000  |     0    |  [000000	0]

-----|----------|----------|----------|----------|------------------------

37  |  700000  |   129    |  000000  |     0    |  [000000	0]

-----|----------|----------|----------|----------|------------------------

38  |  000000  |    0     | -400000  |    125   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

39  |  000000  |    0     | -280000  |    128   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

40  |  000000  |    0     |  500000  |    126   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

41  |  000000  |    0     |  700000  |    129   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

42  |  000000  |    0     |  000000  |    0     | -[000000	0]

-----|----------|----------|----------|----------|------------------------

43  |  000000  |    0     |  000001  |    0     | -[000000	0]

-----|----------|----------|----------|----------|------------------------

44  |  000000  |    0     | -000001  |    0     | -[000000	0]

-----|----------|----------|----------|----------|------------------------

45  |  000000  |    0     |  7fffff  |    254   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

46  |  000000  |    0     | -7fffff  |    254   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

47  |  000000  |    0     |  000000  |    255   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

48  |  000000  |    0     | -000000  |    255   | -[000000	0]

-----|----------|----------|----------|----------|------------------------

49  |  000000  |    0     |  7fffff  |    255   | NAN[7fffff 255]

_____|__________|__________|__________|__________|________________________

BOUNDARY (NEG_ZERO) OPERATIONS

(Negative zero)

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|------------------------

50  | -400000  |   125    | -000000  |    0     |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

51  | -280000  |   128    | -000000  |    0     |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

52  |  500000  |   126    | -000000  |    0     | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

53  |  700000  |   129    | -000000  |    0     | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

54  | -000000  |    0     | -400000  |   125    |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

55  | -000000  |    0     | -280000  |   128    |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

56  | -000000  |    0     |  500000  |   126    | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

57  | -000000  |    0     |  700000  |   129    | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

58  | -000000  |    0     |  000000  |    0     | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

59  | -000000  |    0     |  000001  |    0     | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

60  | -000000  |    0     | -000001  |    0     |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

61  | -000000  |    0     |  7fffff  |   254    | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

62  | -000000  |    0     | -7fffff  |   254    |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

63  | -000000  |    0     |  000000  |   255    | -[000000 	0]

-----|----------|----------|----------|----------|------------------------

64  | -000000  |    0     | -000000  |   255    |  [000000 	0]

-----|----------|----------|----------|----------|------------------------

65  | -000000  |    0     |  7fffff  |   255    |NAN[7fffff  255]

_____|__________|__________|__________|__________|________________________

BOUNDARY (MIN_POS) OPERATIONS

(Minimal Positive Number)

IEEE format for MIN_POS ->  sign = 0,  exponent = 0,  mantissa = 000000..01

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

66  | -400000  |   128    |  000001  |    0     | -[400002	   1]

-----|----------|----------|----------|----------|-----------------------------

67  | -280000  |   128    |  000001  |    0     | -[280001	   1]

-----|----------|----------|----------|----------|-----------------------------

68  |  500000  |   129    |  000001  |    0     |  [500002	   2]

-----|----------|----------|----------|----------|-----------------------------

69  |  700000  |   129    |  000001  |    0     |  [700002	   2]

-----|----------|----------|----------|----------|-----------------------------

70  |  000001  |    0     | -400000  |   128    | -[400002 	   2]

-----|----------|----------|----------|----------|-----------------------------

71  |  000001  |    0     | -280000  |   128    | -[280001	   1]

-----|----------|----------|----------|----------|-----------------------------

72  |  000001  |    0     |  500000  |   127    |  [500002	   0]

-----|----------|----------|----------|----------|-----------------------------

73  |  000001  |    0     |  700000  |   129    |  [700002	   2]

-----|----------|----------|----------|----------|-----------------------------

74  |  000001  |    0     |  000000  |    0     |  [000000	   0]

-----|----------|----------|----------|----------|-----------------------------

75  |  000001  |    0     |  7fffff  |   254    |  [000000 	 128]

-----|----------|----------|----------|----------|-----------------------------

76  |  000001  |    0     | -7fffff  |   254    | -[000000	 128]

-----|----------|----------|----------|----------|-----------------------------

77  |  000001  |    0     |  000000  |   255    |  [000000	 255]

-----|----------|----------|----------|----------|-----------------------------

78  |  000001  |    0     | -000000  |   255    | -[000000	 255]

-----|----------|----------|----------|----------|-----------------------------

79  |  000001  |    0     |  555555  |   255    |Nan[555555	 255]

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (MIN_NEG) OPERATIONS

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|----------------------------

80  | -400000  |   128    | -000001  |    0     |  [400002	   1]

-----|----------|----------|----------|----------|----------------------------

81  | -280000  |   128    | -000001  |    0     |  [280001	   1]

-----|----------|----------|----------|----------|----------------------------

82  |  500000  |   129    | -000001  |    0     | -[500002	   2]

-----|----------|----------|----------|----------|----------------------------

83  |  700000  |   129    | -000001  |    0     | -[700002	   2]

-----|----------|----------|----------|----------|----------------------------

84  | -000001  |    0     | -400000  |   128    |  [400002	   1]

-----|----------|----------|----------|----------|----------------------------

85  | -000001  |    0     | -280000  |   128    |  [280001	   1]

-----|----------|----------|----------|----------|----------------------------

86  | -000001  |    0     |  500000  |   129    | -[500002	   2]

-----|----------|----------|----------|----------|----------------------------

87  | -000001  |    0     |  700000  |   129    | -[700002	   2]

-----|----------|----------|----------|----------|----------------------------

88  | -000001  |    0     |  000000  |    0     | -[000000	   0]

-----|----------|----------|----------|----------|----------------------------

89  | -000001  |    0     |  7fffff  |   254    | -[000000	 128]

-----|----------|----------|----------|----------|----------------------------

90  | -000001  |    0     | -7fffff  |   254    |  [000000	 128]

-----|----------|----------|----------|----------|----------------------------

91  | -000001  |    0     |  000000  |   255    | -[000000	 255]

-----|----------|----------|----------|----------|----------------------------

92  | -000001  |    0     | -000000  |   255    |  [000000	 255]

-----|----------|----------|----------|----------|----------------------------

93  | -000001  |    0     |  400000  |   255    |Nan[400000	 255]

_____|__________|__________|__________|__________|____________________________

BOUNDARY (MAX_POS) OPERATIONS

(Maximum positive number)

IEEE format for MAX_POS ->  sign = 0,  exponent = 254,  mantissa = 11111..11

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

94  | -400000  |   125    |  7fffff  |   254    |  -[3fffff	253]

-----|----------|----------|----------|----------|-----------------------------

95  | -280000  |   125    |  7fffff  |   254    |  -[27ffff	253]

-----|----------|----------|----------|----------|-----------------------------

96  |  500000  |   126    |  7fffff  |   254    |   [4fffff	254]

-----|----------|----------|----------|----------|-----------------------------

97  |  700000  |   126    |  7fffff  |   254    |   [6fffff	254]

-----|----------|----------|----------|----------|-----------------------------

98  |  7fffff  |   254    | -400000  |   125    |  -[3fffff	253]

-----|----------|----------|----------|----------|-----------------------------

99  |  7fffff  |   254    | -280000  |   124    |  -[27ffff	252]

-----|----------|----------|----------|----------|-----------------------------

100 |  7fffff  |   254    |  500000  |   126    |   [4fffff	254]

-----|----------|----------|----------|----------|-----------------------------

101 |  7fffff  |   254    |  700000  |   123    |   [6fffff	251]

-----|----------|----------|----------|----------|-----------------------------

102 |  7fffff  |   254    |  000000  |    0     |   [000000 	  0]

-----|----------|----------|----------|----------|-----------------------------

103 |  7fffff  |   254    |  000001  |    0     |   [000000 	128]

-----|----------|----------|----------|----------|-----------------------------

104 |  7fffff  |   254    | -000001  |    0     |  -[000000	128]

-----|----------|----------|----------|----------|-----------------------------

105 |  7fffff  |   254    |  000000  |   255    |   [000000	255]

-----|----------|----------|----------|----------|-----------------------------

106 |  7fffff  |   254    | -000000  |   255    |  -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

107 |  7fffff  |   254    |  400000  |   255    |   [400000	255]

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (MAX_NEG) OPERATIONS

(Maximum negative number)

IEEE format for MAX_POS ->  sign = 1,  exponent = 254,  mantissa = 11111..11

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

108 | -400000  |   125    | -7fffff  |   254    |   [3fffff	253]

-----|----------|----------|----------|----------|-----------------------------

109 | -280000  |   126    | -7fffff  |   254    |   [27ffff	254]

-----|----------|----------|----------|----------|-----------------------------

110 |  500000  |   126    | -7fffff  |   254    |  -[4fffff	254]

-----|----------|----------|----------|----------|-----------------------------

111 |  700000  |   122    | -7fffff  |   254    |  -[6fffff	250]

-----|----------|----------|----------|----------|-----------------------------

112 | -7fffff  |   254    | -400000  |   125    |   [3fffff	253]

-----|----------|----------|----------|----------|-----------------------------

113 | -7fffff  |   254    | -280000  |   121    |   [27ffff	249]

-----|----------|----------|----------|----------|-----------------------------

114 | -7fffff  |   254    |  500000  |   126    |  -[4fffff	254]

-----|----------|----------|----------|----------|-----------------------------

115 | -7fffff  |   254    |  700000  |   124    |  -[6fffff	252]

-----|----------|----------|----------|----------|-----------------------------

116 | -7fffff  |   254    |  000000  |    0     |  -[000000	  0]

-----|----------|----------|----------|----------|-----------------------------

117 | -7fffff  |   254    |  000001  |    0     |  -[000000 	128]

-----|----------|----------|----------|----------|-----------------------------

118 | -7fffff  |   254    | -000001  |    0     |   [000000 	128]

-----|----------|----------|----------|----------|-----------------------------

119 | -7fffff  |   254    |  000000  |   255    |  -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

120 | -7fffff  |   254    | -000000  |   255    |   [000000	255]

-----|----------|----------|----------|----------|-----------------------------

121 | -7fffff  |   254    |  400000  |   255    |   [400000	255]

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (POS_INF) ADD OPERATIONS

(Positive infinity)

IEEE format for POS_INF ->  sign = 0,  exponent = 255,  mantissa = all 0's

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

122 | -400000  |   125    |  000000  |   255    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

123 | -280000  |   128    |  000000  |   255    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

124 |  500000  |   126    |  000000  |   255    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

125 |  700000  |   129    |  000000  |   255    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

126 |  000000  |   255    | -400000  |   125    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

127 |  000000  |   255    | -280000  |   128    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

128 |  000000  |   255    |  500000  |   126    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

129 |  000000  |   255    |  700000  |   129    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

130 |  000000  |   255    |  000000  |    0     |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

131 |  000000  |   255    |  000001  |    0     |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

132 |  000000  |   255    | -000001  |    0     | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

133 |  000000  |   255    |  7fffff  |   254    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

134 |  000000  |   255    | -7fffff  |   254    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

135 |  000000  |   255    |  000000  |   255    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

136 |  000000  |   255    | -000000  |   255    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

137 |  000000  |   255    |  7fffff  |   255    |  [7fffff 	255]

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (NEG_INF) OPERATIONS

(Negative infinity)

IEEE format for NEG_INF ->  sign = 1,  exponent = 255,  mantissa = all 0's

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

138 | -400000  |   125    | -000000  |   255    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

139 | -280000  |   128    | -000000  |   255    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

140 |  500000  |   126    | -000000  |   255    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

141 |  700000  |   129    | -000000  |   255    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

142 | -000000  |   255    | -400000  |   125    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

143 | -000000  |   255    | -280000  |   128    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

144 | -000000  |   255    |  500000  |   126    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

145 | -000000  |   255    |  700000  |   129    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

146 | -000000  |   255    |  000000  |    0     | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

147 | -000000  |   255    |  000001  |    0     | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

148 | -000000  |   255    | -000001  |    0     |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

149 | -000000  |   255    |  7fffff  |   254    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

150 | -000000  |   255    | -7fffff  |   254    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

151 | -000000  |   255    |  000000  |   255    | -[000000	255]

-----|----------|----------|----------|----------|-----------------------------

152 | -000000  |   255    | -000000  |   255    |  [000000	255]

-----|----------|----------|----------|----------|-----------------------------

153 | -000000  |   255    |  7ffffc  |   255    |  [7ffffc	255]

_____|__________|__________|__________|__________|_____________________________

BOUNDARY (NAN) OPERATIONS

(Not a number)

IEEE format for NAN ->  sign = 0 or 1,  exponent = 255,  mantissa = not all 0's

|       OPERAND 1     |       OPERAND 2     |

TEST |                     |                     |       OPERATION

#  |SIGN/MANT | EXPONENT |SIGN/MANT | EXPONENT |

-----|----------|----------|----------|----------|-----------------------------

154 | -400000  |   125    |  3fffff  |   255    |   [3fffff	255]

-----|----------|----------|----------|----------|-----------------------------

155 | -280000  |   128    |  2aaaaa  |   255    |   [2aaaaa	255]

-----|----------|----------|----------|----------|-----------------------------

156 |  500000  |   126    |  3fffff  |   255    |   [3fffff    255]

-----|----------|----------|----------|----------|-----------------------------

157 |  700000  |   129    |  2aaaaa  |   255    |   [2aaaaa  	255]

-----|----------|----------|----------|----------|-----------------------------

158 |  3fffff  |   255    | -400000  |   125    |   [3fffff 	255]

-----|----------|----------|----------|----------|-----------------------------

159 |  2aaaaa  |   255    | -280000  |   128    |   [2aaaaa	255]

-----|----------|----------|----------|----------|-----------------------------

160 |  3fffff  |   255    |  500000  |   126    |   [3fffff 	255]

-----|----------|----------|----------|----------|-----------------------------

161 |  2aaaaa  |   255    |  700000  |   129    |   [2aaaaa 	255]

-----|----------|----------|----------|----------|-----------------------------

162 |  3fffff  |   255    |  000000  |    0     |   [3fffff	255]

-----|----------|----------|----------|----------|-----------------------------

163 |  2aaaaa  |   255    |  000001  |    0     |   [2aaaaa  	255]

-----|----------|----------|----------|----------|-----------------------------

164 |  3fffff  |   255    | -000001  |    0     |   [3fffff 	255]

-----|----------|----------|----------|----------|-----------------------------

165 |  2aaaaa  |   255    |  7fffff  |   254    |   [2aaaaa  	255]

-----|----------|----------|----------|----------|-----------------------------

166 |  3fffff  |   255    | -7fffff  |   254    |   [3fffff	255]

-----|----------|----------|----------|----------|-----------------------------

167 |  2aaaaa  |   255    |  000000  |   255    |   [2aaaaa  	255]

-----|----------|----------|----------|----------|-----------------------------

168 |  3fffff  |   255    |  2aaaaa  |   255    |   [3fffff  	255]

-----|----------|----------|----------|----------|-----------------------------

169 |  2aaaaa  |   255    |  7ffffc  |   255    |   [2aaaaa	255]

_____|__________|__________|__________|__________|_____________________________